| Steps | Detergent | Dilute | Diluted Coverage | TASKI Pads & Brushes |
| --- | --- | --- | --- | --- |
| Initial Cleaning | Buckeye Blue TASKI profi™ | 2 oz / gal water 10 oz / gal water | 180 to 360 sq ft / gal | Soft nylon brush |
| Neutral Cleaner | Buckeye Blue TASKI profi™ | 4 oz / gal water 10 oz / gal water | 180 to 360 sq ft / gal | Contact pad, purple pad or soft nylon brush |
| Heavy Soil Cleaner | Buckeye Blue TASKI profi™ | 8 oz / gal water 10 oz / gal water | 180 to 360 sq ft / gal | Contact pad, purple pad or soft nylon brush |

Broom              Wet Mop          Wet/Dry Vacuum           Auto Scrubber             Buffer                 SteamVac          Rotowash

I. Cleaning Instructions

Initial Cleaning

- Remove all surface soil, debris, sand and grit with a vacuum.

- Scrub floor with a neutral detergent.

- Pick up solution with a wet vacuum, rinse with clean water and allow to dry thoroughly.

- This process may be done in one or two steps with a Rotowash or SteamVac.

Daily / Regular Cleaning

- Sweep or vacuum floor to remove surface soil, debris, sand and grit.

- Damp mop the floor.

- This process may be done in one or two steps with a Rotowash or SteamVac.

- Allow floor to dry thoroughly.

Interim Maintenance

- Vacuum floor thoroughly.

- Aggressively scrub the floor with a neutral detergent. Use the rotary scrubber or Rotowash.

- Vacuum any remaining soiled solution with a wet/dry vacuum.

- Allow floor to dry thoroughly.

Restorative Maintenance

- Vacuum floor thoroughly.

- Scrub using a rotary scrubber or a Rotowash and neutral cleaner.

- Pick up solution with wet vacuum or SteamVac.

- Allow floor to dry thoroughly.

Page 1 of 2

II. Equipment Specifications

| SteamVac Machine | Similar to any Hoover SteamVac model with scrubber and vacuum. |
| --- | --- |
| Rotowash Machine | Model R-4 with a 15 inch cleaning width for large areas. The smaller R-3 model with an 11 inch cleaning width works well around equipment. Roto Wash is available from the manufacturer at www.rewhittaker.com/qstore. |
| Power Scrubber | 17 inch rotary floor buffer with mounted detergent tank and feed line to the brush. A circular brush attachment should be used. |
| Wet/Dry Shop Vacuum Unit | Power Unit: Minimum 1.7 hp commercial unit, 7.0 amp, 120vlt A.C., 50/60 Hz two stage bypass motor Tank Capacity: 10-25 gallon, lined stainless stel or polypropylene. Accessories: Extension wand with a 6 to 12 inch pickup nozzle, crevice tool, heavy-duty extension cord. |
| Detergent | Commercial/Industrial non-petroleum based neutral detergent. - Buckeye Blue - TASKI profi™, TASKI ice it™ or their equivalent. |

Important Maintenance tips

- Use high CFM vacuum to pick up dust.

- For large areas, use the Rotowash Model R-4B.

- Dilute the detergents with water per the manufacturers instructions; use higher concentrations for stubborn stains or hard to clean areas.

- Rinse, wet vacuum or mop up any detergent residue when cleaning the floor.

- For gum, use generic gum remover, however, any solvent-based product should be rinsed well with water to avoid damaging floor.

- DO NOT use an Auto Scrubber with a squeegee attachment. It tends to not pick up all of the dirty water solution and leaves a film of diluted dirty water on the tiles.

- DO NOT use petroleum or citrus based detergents.

Contact Buckeye Blue Customer Care at 800-321-2583 for a distributor in your area.

Contact TASKI Customer Care at 800-862-3400 for a distributor in your area.

Page 2 of 2                                                                                                                        888-479-101